IOT Based Air Pollution Monitoring System

- Abstract:

The level of pollution is increasing rapidly due to factors like industrialization, urbanization, increase in population degrading the air quality. The Air Pollution Monitoring System is used to monitor the Air Quality over a web server using Internet. It will trigger an alarm when the air quality goes beyond a certain level, meaning when there are sufficient amount of harmful gases present in the air like CO2, smoke, alcohol, NH3 and alarm. It will show the air quality in PPM on the cloud website so that air pollution can be monitored very easily. The system uses sensors such as MQ135 and DHT11 sensor which are connected to NodeMCU ESP 8266 Microcontroller. NodeMCU ESP 8266 is an open-source Lua based firmware and development board specially targeted for IOT based applications. It includes firmware that runs on ESP 8266 Wifi SoC from Espressif Systems. The MQ135 sensor can detect gasses like Ammonia, Sulphur, CO2 and smoke. The DHT11 sensor is used to detect the temperature and the amount of humidity present in the air. The NodeMCU microcontroller is initially uploaded with the code, which collects data and sends it to the cloud using a local Wifi Hotspot. The real-time data collected from the sensors is sent through the internet to the Arduino cloud IOT using which we can plot data sets and analyze data.

- Introduction:

Air is getting polluted because of release of toxic gases by industries, vehicle emissions

and increased concentration of harmful gases and particulate matter in the atmosphere.

The level of pollution is increasing rapidly due to factors like industries, urbanization,

increasing in population, vehicle use which can affect human health. Particulate matter

is one of the most important parameter having the significant contribution to the

increase in air pollution.

The Air pollution and monitoring system will show the air on webpage so that it can monitor it very easily. LPG sensor is added in this system which is used mostly in houses. The system will show temperature and humidity. The system can be installed anywhere but mostly in industries and houses where gases are mostly to be found. LPG gas is detected using MQ135 sensor is used for monitoring Air Quality as it detects most harmful gases and can measure their amount accurately. In this IOT project, it can monitor the pollution level from anywhere using your computer or mobile.

- Literature Review:

Air pollution, the biggest problem of every developing or developed nation. Health problems have been growing at a faster pace especially in urban areas of developing countries where industrialization and growing number of vehicles leads to release of lot of gaseous pollutants.

The difficulty of the conventional monitoring instruments is their large size, heavy weight and extraordinary costlier. These lead to inadequate deployment of the monitoring stations. In order to be effective, the locations of the monitoring stations need careful placement because the air pollution situation in urban areas is highly related to human and location-independent. Harmful effects of pollution include mild allergic reactions such as irritation of the throat, eyes and nose as well as some serious problems like bronchitis, heart diseases, pneumonia, lung and aggravated asthma.

IOT based air pollution monitoring system monitors the air quality over a web server using Internet when there are sufficient amount of harmful gases present in the air like CO2, smoke, alcohol, benzene, NH3, LPG and NOx. These gases are measured in PPM and are displayed which can help us to track and monitor and them easily.

- IOT Based Air Pollution Monitoring System Using Node MCU Arduino

Poonam Pal, Ritik Gupta, Sanjana Tiwari, Ashutosh Sharma

In this project we are going to make an IOT Based Air Pollution Monitoring System in which we will monitor the Air Quality over a web server using internet and will trigger a alarm when the air quality goes down beyond a certain level, means when there are sufficient amount of harmful gases are present in the air like CO2, smoke, alcohol, benzene and NH3. It will show the air quality in PPM on the LCD and as well as on webpage so that we can monitor it very easily. In this IOT project, you can monitor the pollution level from anywhere using your computer or mobile.

- IOT Based Air Pollution Monitoring System

Harsh N. Shah 1 , Zishan Khan 2 , Abbas Ali Merchant 3 , Moin Moghal 4 , Aamir Shaikh 5, Priti Rane 6 1, 2, 3, 4,5Student, Diploma in Computer Engineering, BGIT, Mumbai Central, India 6 Assistant Professor, BGIT, Mumbai Central India Air pollution is the biggest problem of every nation, whether it is developed or developing. Health problems have been growing at faster rate especially in urban areas of developing countries where industrialization and growing number of vehicles leads to release of lot of gaseous pollutants. IOT Based Air Pollution Monitoring System monitors the Air quality over a web server using Internet and will trigger an alarm when the air quality goes down beyond a certain threshold level, means when there are sufficient amount of harmful gases present in the air like CO2, smoke, alcohol, benzene, NH3, LPG and NOx. It will show the air quality in PPM on the LCD and as well as on webpage.

3) IOT Based Air Quality Monitoring System

Ch.V.Saikumar[1] M.Reji[2] P.C.Kishoreraja[3] Department of ECE ,Saveetha School of Engineering Saveetha University, Chennai, India

The main objective of this project is to monitor the air eminence in industrial and urban areas. The proposed outline includes a set of gas sensors (CO, and NO2) that are positioned on masses and structure of a IOT (Internet of things) and a dominant server to support both short-range real time incident management and a continuing deliberate planning. In this Arduino platform is used to communicate the data simply and quickly. WSN (Wireless sensor network) acts as the trans receiver. This provide a real-time low rate monitoring system over the use of low rate, low information rate, and little control wireless communication technology. The projected monitoring system can be transferred to or shared by different applications. Through IOT we can able to visualize the values from the globe.

- Requirements:

i) Hardware Requirements:

1. MQ135 Gas Sensor

2. NodeMCU ESP8266

3. Humidity Sensor DHT11

4. Breadboard

5. Connections & Data Cable

ii) Software Requirements:

1. Embedded C Language

2. Adruino IoT Cloud

- Working:

The NodeMCU ESP 8266 has 17 GPIOs, SPI, I2C (implemented on software), I2S interfaces with DMA, UART and 10-bit ADC. To attatch the given components to the NodeMCU the circuit has to be connected as shown below.

As given in the figure, the DHT11 sensor consists of signal, VCC and GND pins. The signal pin is connected to one of the GPIO 2/D4  (General Purpose Input\Output pins), the GND is connected to GND of NodeMCU and the VCC is connected to 3V3 (3.3V supply pin).

Similarly, the MQ135 gas sensor consists of VCC, GND, Digital Out and Analog Out. The VCC is connected to the 3V3 port present on the NodeMCU, GND to GND of the NodeMCU and the analog out to the A0 port of the NodeMCU.

In the beginning, the NodeMCU is operated in station mode and waits for an active LAN to connect. After connecting to a LAN, it will establish a connection with the Arduino cloud by which we can visit the webpage and monitor the data. The code starts to fetch data from the sensors and assigns it to the variables. Once the variable is assigned a value, it sends it to the Arduino cloud. The data is then stored, monitored and displayed in the graphs and fields provided.

To check the working of the MQ sensor, you can just test it by providing any gas near to it. We could just use a lighter to test the working of the MQ sensor.

To test the working of DHT11 we could just blow air at it to check and observe the shift in the Temperature and Humidity. The live changes are shown in graphs and the meter shows the live current value.

- Components:

i. MQ135 Gas Sensor

The MQ135 sensor can sense NH3, NOx, alcohol, Benzene, smoke, CO2 and some other gases. It gives the output in form of voltage levels.

ii. NodeMCU ESP8266

The ESP8266 is a low-cost Wi-Fi chip with full TCP/IP stack and MCU (microcontroller unit) capability. It runs on 3.3V and gives our system access to Wi-Fi or internet.

iii. DHT11

The DHT11 is a basic, ultra low-cost digital temperature and humidity sensor. It uses a capacitive humidity sensor and a thermistor to measure the surrounding air, and spits out a digital signal on the data pin.

- Advantages:

1. Easy to install

2. Accurate Pollution Monitoring

3. Remote Location Monitoring

- Applications:

1. Indoor Air Quality Monitoring

2. Making data available to users

- References:

[1] https://www.ijser.org/researchpaper/IOT-Based-Air-Pollution-Monitoring-System.pdf

[2] WHO, Global Environmental Change, World Health Organization, Geneva, Switzerland, 2005.

[3] Air Resource Management Centre, Vehicle-related air pollutants and public health, Ministry of Environment and Natural Resources, Sri Lanka, May 2003, pp. 611.

[4] Urban Air Quality Management Project, Impact of fuel changes on exhaust and evaporative emissions, Ministry of Environment and Natural Resources, Sri Lanka, May 2003, pp. 7585.

[5] S. Kelly, N. Suryadevara, and S. Mukhopadhyay, Towards the Implementation of IoT for

Environmental Condition Monitoring in Homes, Sensors Journal, IEEE, vol. 13, no. 10, pp. 38463853, Oct. 2013.

[6] H. Yang, Y. Qin, G. Feng, and H. Ci, Online Monitoring of Geological CO2 Storage and Leakage

Based on Wireless Sensor Networks, Sensors Journal, IEEE, vol. 13, no. 2, pp. 556562, Feb. 2013.

[7] IST, Gas sensor calibration, International Sensor Technology, CA, USA, Ch. 11, pp. 161173